SmartCare Hospital

Network Design & Configuration Documentation

1.  Project Information

| Field | Detail |
| --- | --- |
| Project Title | SmartCare Hospital Network Design |
| Platform | Cisco Packet Tracer |
| Architecture | Hierarchical - Core / Distribution / Access |
| Sites | Headquarters + Branch Office |

2.  Network Architecture Overview

Core / WAN Layer

- Provides backbone connectivity and internet access via Dual ISP failover.

- HQ: One Core Router (Dual ISP, ). Branch: One Branch Router (Dual ISP).

Distribution Layer

- Handles inter-VLAN routing, DHCP relay (Branch), and gateway redundancy via HSRP.

- HQ: one L3 switches - Dist-SW1.

- Branch:one L3 switches - Dist-SW1.

Access Layer

- Connects all end devices to the network via L2 access switches with PortFast and BPDU Guard.

3.  Device Inventory

3.1  Headquarters Devices

| Device | Hostname | Role | Location |
| --- | --- | --- | --- |
| Router | SmartCare-HQ-Core-Router | WAN Gateway / NAT / Dual ISP | HQ |
| L3 Switch | SmartCare-HQ-Dist-SW1 | Distribution - HSRP Active | HQ |
| 10 L2 Switches | HQ-department-SW | Access Layer | HQ |

3.2  Branch Office Devices

| Device | Hostname | Role | Location |
| --- | --- | --- | --- |
| Router | Branch-Router | WAN Gateway / NAT / Dual ISP | Branch |
| L3 Switch | SmartCare-Branch-Dist-SW1 | Distribution - HSRP Active | Branch |
| 10 L2 Switch | SmartCare-Branch-department-SW | Access Layer | Branch |

- Credentials & Access Details

- All passwords are shared across all devices .

| Access Method | Line / Setting | Value |
| --- | --- | --- |
| Console | line console 0 | SmartCare@2026 |
| VTY (SSH) | line vty 0 4 | SmartCare@2026 |
| Enable Secret | enable secret | SmartCare@2026 |
| SSH Version | ip ssh version | 2 (RSA 1012-bit) |
| Domain | ip domain-name | smartcare.local |

5.  WAN & Dual ISP Configuration

5.1  HQ - ISP Interface Assignments

| ISP | Interface | HQ Router IP | ISP Gateway | Role |
| --- | --- | --- | --- | --- |
| ISP 1 | GigabitEthernet0/0 | 100.0.0.2/30 | 100.0.0.1 | Primary (AD 5) |
| ISP 2 | GigabitEthernet0/1 | 200.0.0.2/30 | 200.0.0.1 | Backup (AD 10) |

5.2  Branch - ISP Interface Assignments

| ISP | Interface | Branch Router IP | ISP Gateway | Role |
| --- | --- | --- | --- | --- |
| ISP 1 | GigabitEthernet0/0 | 203.0.113.6/30 | 203.0.113.5 | Primary (AD 1) |
| ISP 2 | GigabitEthernet0/1 | 198.51.100.6/30 | 198.51.100.5 | Backup (AD 10) |

5.3  LAN Links - Router to Distribution Switches

| Site | Interface | Connects To | IP Address |
| --- | --- | --- | --- |
| HQ | GigabitEthernet0/2 | HQ-Dist-SW1 | 10.0.0.1/30 |
| Branch | GigabitEthernet0/2 | Branch-Dist-SW1 | 10.1.0.1/30 |

6.  VLAN Design & IP Addressing

Base Network: 192.168.0.0/16   |   HQ VLANs: 192.168.10-110.0/24   |   Branch VLANs: 192.168.120-220.0/24

| VLAN | Name | Department | HQ Network | HQ GW | Branch Network | Branch GW |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | admin | Administration | 192.168.10.0/24 | 192.168.10.1 | 192.169.10.0/24 | 192.169.10.1 |
| 20 | finance | Finance | 192.168.20.0/24 | 192.168.20.1 | 192.169.20.0/24 | 192.169.20.1 |
| 30 | hr | HR | 192.168.30.0/24 | 192.168.30.1 | 192.169.30.0/24 | 192.169.30.1 |
| 40 | reception | Reception | 192.168.40.0/24 | 192.168.40.1 | 192.169.40.0/24 | 192.169.40.1 |
| 50 | guest | Guest | 192.168.50.0/24 | 192.168.50.1 | 192.169.50.0/24 | 192.169.50.1 |
| 60 | it | IT / DHCP Server | 192.168.60.0/24 | 192.168.60.1 | 192.169.60.0/24 | 192.169.60.1 |
| 70 | emergency | Emergency | 192.168.70.0/24 | 192.168.70.1 | 192.169.70.0/24 | 192.169.70.1 |
| 80 | pharmacy | Pharmacy | 192.168.80.0/24 | 192.168.80.1 | 192.169.80.0/24 | 192.169.80.1 |
| 90 | dr | Doctors & Nurses | 192.168.90.0/24 | 192.168.90.1 | 192.169.90.0/24 | 192.169.90.1 |
| 100 | lab | Labs | 192.168.100.0/24 | 192.168.100.1 | 192.169.100.0/24 | 192.169.100.1 |
| 110 | management | Management | 192.168.110.0/24 | 192.168.110.1 | 192.169.110.0/24 | 192.169.110.1 |

VLAN 50 (Guest) uses DNS 8.8.8.8 and is blocked from all internal hospital subnets by ACL on all distribution switches.

7.  DHCP Configuration

7.1  HQ DHCP (on Dist-SW1)

DHCP pools are configured on a dedicated DHCP SERVER. Each VLAN has a dedicated dhcp server pool.

| Pool Name | Network | Default Gateway | DNS Server | Start IP |
| --- | --- | --- | --- | --- |
| VLAN10 | 192.168.10.0/24 | 192.168.10.1 | 192.168.60.10 | 192.168.10.10 |
| VLAN20 | 192.168.20.0/24 | 192.168.20.1 | 192.168.60.10 | 192.168.20.10 |
| VLAN30 | 192.168.30.0/24 | 192.168.30.1 | 192.168.60.10 | 192.168.30.10 |
| VLAN40 | 192.168.40.0/24 | 192.168.40.1 | 192.168.60.10 | 192.168.40.10 |
| VLAN50 | 192.168.50.0/24 | 192.168.50.1 | 8.8.8.8 | 192.168.50.10 |
| VLAN60 | 192.168.60.0/24 | 192.168.60.1 | 192.168.60.10 | 192.168.60.10 |
| VLAN70 | 192.168.70.0/24 | 192.168.70.1 | 192.168.60.10 | 192.168.70.10 |
| VLAN80 | 192.168.80.0/24 | 192.168.80.1 | 192.168.60.10 | 192.168.80.10 |
| VLAN90 | 192.168.90.0/24 | 192.168.90.1 | 192.168.60.10 | 192.168.90.10 |
| VLAN100 | 192.168.100.0/24 | 192.168.100.1 | 192.168.60.10 | 192.168.100.10 |
| VLAN110 | 192.168.110.0/24 | 192.168.110.1 | 192.168.60.10 | 192.168.110.10 |

7.2  Branch DHCP

The Branch uses a dedicated DHCP/DNS server in VLAN 60. distribution switches relay DHCP requests via ip helper-address 192.168.170.10 on every SVI.

| Pool Name | Network | Default Gateway | DNS Server | Start IP |
| --- | --- | --- | --- | --- |
| Admin | 192.169.10.0/24 | 192.169.x.1 | 192.169.60.10 | 192.168.x.10 |
| Finance | 192.169.20.0/24 |  |  |  |
| HR | 192.169.30.0/24 |  |  |  |
| Reception | 192.169.40.0/24 |  |  |  |
| Guest | 192.169.50.0/24 |  |  |  |
| IT | 192.169.60.0/24 |  |  |  |
| Emergency | 192.169.70.0/24 |  |  |  |
| Pharmacy | 192.169.80.0/24 |  |  |  |
| Doctors | 192.169.90.0/24 |  |  |  |
| Lab | 192.169.100.0/24 |  |  |  |

8.  Distribution Switch Interface Map

Applies to both HQ and Branch distribution switches .

| Interface | Description | IP / Mode | Connected To |
| --- | --- | --- | --- |
| GigabitEthernet1/0/1 | Routed Uplink to Router | 10.x.x.2/30 (SW1) ¦ | Site Router |
| fastEthernet0/1-10 | Access Trunk | Trunk - VLANs 10-130 | Access SW |

9.  Access Switch Port Assignments

Same port template applies to HQ and Branch access switches. Adjust port ranges to match physical cabling.

| switch | port | vlan | PortFast | BPDU Guard |
| --- | --- | --- | --- | --- |
| admin | Fa0/2 trunk Fa0/3 vlan120,130 Fa0/4-15 vlan 10 |  | Enabled | Enabled |
| finance | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 20 | Enabled | Enabled |
| hr | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 30 | Enabled | Enabled |
| reception | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 40 | Enabled | Enabled |
| guest | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 50 | Enabled | Enabled |
| IT | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 60 | Enabled | Enabled |
| emergency | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 70 | Enabled | Enabled |
| pharmacy | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 80 | Enabled | Enabled |
| doctors | Fa0/1-2 Fa0/3 Fa0/4-15 | Trunk 120,130 90 | Enabled | Enabled |
| lab | Fa0/1-2 Fa0/3 Fa0/4-150 | Trunk 120,130 100 | Enabled | Enabled |

- Gi0/1 -> Trunk to Dist-SW1 | VLANs: 10,20,30,40,50,60,70,80,90,100,110

- Management SVI: HQ = 192.168.110.11 | Branch = 192.168.220.11 | GW = x.x.x.1

10.  Routing - OSPF Configuration

| Device | Router-ID | OSPF Area | Notes |
| --- | --- | --- | --- |
| HQ-Core-Router | 1.1.1.1 | Area 0 (Backbone) | default-information originate always; WAN interfaces passive |
| HQ-Dist-SW1 |  | Area 0 | Advertises all HQ VLAN networks |
| Branch-Router | 2.2.2.2 | Area 1 (Branch) | default-information originate always; WAN interfaces passive |
| Branch-Dist-SW1 |  | Area 1 | Advertises all Branch VLAN networks |

- HQ is OSPF Area 0 (backbone). Branch is Area 1, connected to Area 0 via WAN/MPLS link.

- WAN interfaces (Gi0/0, Gi0/1) are passive on both routers - no OSPF hellos sent to ISPs.

11.  Spanning Tree Configuration

| Device | Mode | STP Role | VLANs |
| --- | --- | --- | --- |
| HQ-Dist-SW1 | Rapid-PVST | Root Primary | 10,20,30,40,50,60,70,80,90,100,110 |
| Branch-Dist-SW1 | Rapid-PVST | Root Primary | 10,20,30,40,50,60,70,80,90,100,110 |
| All Access SWs | Rapid-PVST | Non-Root | PortFast + BPDU Guard enabled on all access ports |

12.  Security Configuration

12.1  Guest VLAN Isolation (ACL)

| ACL Name | Applied On | Direction | Effect |
| --- | --- | --- | --- |
| GUEST-ISOLATION | HQ Vlan50 SVI | inbound | Blocks Guest (192.168.50.0/24) from all internal subnets; permits internet only |
| BRANCH-GUEST-ISOLATION | Branch Vlan50 SVI | inbound | Blocks Guest (192.168.160.0/24) from all internal subnets; permits internet only |

12.2  Security Hardening (All Devices)

- service password-encryption enabled on all devices

- no ip domain-lookup prevents CLI DNS delay

- Banner motd: Authorized Access Only - SmartCare Hospital

- exec-timeout 10 0 on all console and VTY lines (10-min idle timeout)

- logging synchronous on console lines

- SSH Version 2 with RSA 2048-bit keys; Telnet disabled